Microhardness data normalizition modeling

import numpy as np
import pandas as pd
import matplotlib.pyplot as plt
from sklearn.preprocessing import StandardScaler
from sklearn.gaussian_process import GaussianProcessRegressor
from sklearn.gaussian_process.kernels import RBF,ConstantKernel as c
from sklearn.gaussian_process.kernels import WhiteKernel
from sklearn.model_selection import ShuffleSplit
from sklearn.metrics import mean_absolute_error, mean_squared_error
from mpl_toolkits.mplot3d import Axes3D
import seaborn as sns
from matplotlib.colors import LogNorm
from matplotlib import cm
from matplotlib.pyplot import MultipleLocator
import xlwt
import warnings

sns.set()
warnings.filterwarnings('ignore')

plt.rcParams['font.sans-serif'] = 'SimSun'
plt.rcParams['axes.unicode_minus'] = False

font1 = {
    'family': 'SimSun',
    'weight': 'bold',
    'size': 18
}
font2 = {
    'family': 'Times New Roman',
    'weight': 'normal',
    'size': 15
}
font3 = {
    'family': 'Times New Roman',
    'weight': 'normal',
    'size': 10
}

Load data

filepath = r'C:\Users\Administrator\Desktop\experimental_data\30%-microharness.xlsx'# dataset address
df = pd.read_excel(filepath, skiprows=0)
df.head()

power

feed rate

microhardness

0

700

1.0

466

1

800

1.0

503

2

900

1.0

501

3

1000

1.0

505

4

1100

1.0

520

# remove missing values
df.dropna(axis=0, inplace=True)
df.shape

(24, 3)

# examine microhardness distribution 
sns.scatterplot(x='power', y='feed rate', size='microhardness',\
               data=df)

<Axes: xlabel='power', ylabel='feed rate'>

png

sns.set_style('darkgrid')
fig = plt.figure(figsize=(12, 10))
ax = Axes3D(fig)
fig.add_axes(ax)
ax_obj = ax.scatter(df['power'], df['feed rate'],c=df['microhardness'],\
                   cmap=cm.plasma)
plt.colorbar(ax_obj, shrink=0.7)
ax.set_title('Microhardness Distribution', fontdict=font1)
ax.set_xlabel('\nPower(w)', fontdict=font1)
ax.set_ylabel('\nFeed rate(rad/min)', fontdict=font1)
plt.tick_params(labelsize=13)
plt.show()

png

data = df.pivot(index='power', columns='feed rate', values='microhardness').T
data.sort_index(ascending=False, inplace=True)
data

power

700

800

900

1000

1100

1200

feed rate

1.00

466

503

501

505

520

495

0.85

433

500

516

501

521

531

0.70

474

521

533

542

562

537

0.55

485

518

519

535

559

590

# plot the color map
fig= plt.subplots(figsize=(10,6))
ax_obj1 = sns.heatmap(data, annot=True, fmt="d", linewidths=.6)
plt.title('Microhardness distribution', fontdict=font1)
plt.xlabel('\nLaser power(W)', fontdict=font1)
plt.ylabel('\nPowder feed rate(rad/s)', fontdict=font1)
a, b = data.shape
plt.show()

png

Data normalization, 0 mean, 1 variance

scaler_X = StandardScaler().fit(df[['power', 'feed rate']])
scaler_X.mean_
new_X = scaler_X.transform(df[['power', 'feed rate']])

scaler_Y = StandardScaler().fit(df[['microhardness']])
scaler_Y.mean_
scaler_Y.scale_
new_Y = scaler_Y.transform(df[['microhardness']])

# standardized data table
new_df = pd.DataFrame()
new_df['power'] = new_X[:, 0].ravel()
new_df['feed rate'] = new_X[:, 1].ravel()
new_df['microhardness'] = new_Y.ravel()

new_df.head()

power

feed rate

microhardness

0

-1.46385

1.341641

-1.525692

1

-0.87831

1.341641

-0.380456

2

-0.29277

1.341641

-0.442360

3

0.29277

1.341641

-0.318551

4

0.87831

1.341641

0.145734

# check the distribution of the standardized data
sns.set_style('darkgrid')
fig = plt.figure(figsize=(12, 10))
ax = Axes3D(fig)
fig.add_axes(ax)
ax_obj = ax.scatter(new_df['power'], new_df['feed rate'],c=new_df['microhardness'],\
                   cmap=cm.plasma)
plt.colorbar(ax_obj, shrink=0.7)
ax.set_title('Distribution After Standardization', fontdict=font1)
ax.set_xlabel('Power', fontdict=font1)
ax.set_ylabel('Feed rate', fontdict=font1)
plt.tick_params(labelsize=13)
plt.show()

png

Cross-validation model evaluation

approximately 10-fold

shuffle = ShuffleSplit(n_splits=10, test_size=4, random_state=42)
index = np.arange(16)
for train_index, test_index in shuffle.split(index):
    print(train_index)
    print(test_index)

[13 11  8  9  2 15  4  7 10 12  3  6]
[ 0  1  5 14]
[ 9 12 10  3  0 15 14  5 11  7  1  4]
[ 6  2  8 13]
[ 4 14  0  7  3  6  2 12 11  9 15 10]
[ 1 13  8  5]
[12  7  4 10 11  9 13 15  3  1  6  8]
[ 0  5  2 14]
[ 5  8  9 13  1  3  0  2  7 12 11  6]
[14 10  4 15]
[ 0 10  6  5  7  3 13 15 11  9  1 14]
[ 2  8  4 12]
[ 5 15  6 11  0 10  8  9  7  4  1 12]
[14 13  2  3]
[ 8  3 10  1  6  4 14 12 15  7  0  2]
[11  9 13  5]
[13  0  9 10  5 12  2  4  7 14  6 15]
[ 3  1 11  8]
[ 3 14  7 10 15  8 11  6  9  4  0  2]
[12 13  1  5]

x2 = [420, 455, 490, 525, 560, 595]
sns.set_style('darkgrid')

def cv_loop2(reg):
    mses = []
    maes = []
    maes_relative = []
    i = 1
    plt.figure(figsize=(8, 6))
    for train_index, test_index in shuffle.split(index):
        # extract the training and test datasets      
        x_train = new_df[['power', 'feed rate']].iloc[train_index]
        y_train = new_df['microhardness'].iloc[train_index]
        x_test = new_df[['power', 'feed rate']].iloc[test_index]
        y_test = new_df['microhardness'].iloc[test_index]
        # model prediction and training       
        reg.fit(x_train, y_train)
        print(reg.log_marginal_likelihood())
        y_predict = reg.predict(x_test).reshape(-1,1)
        y_test_arr = np.array(y_test.tolist())
        y_test_arr = y_test_arr.reshape(-1,1)
        y_test_inverse = scaler_Y.inverse_transform(y_test_arr)
        y_predict_inverse = scaler_Y.inverse_transform(y_predict)
        plt.scatter(y_test_inverse, y_predict_inverse)
        mse = mean_squared_error(y_test_inverse, y_predict_inverse, squared=False)
        mses.append(mse)
        mae = mean_absolute_error(y_test_inverse, y_predict_inverse)
        maes.append(mae)
        maes_relative.append(mae/np.mean(y_test_inverse))
        i += 1
    plt.plot(x2, x2, 'r-')
    plt.xlim(420, 595)
    plt.ylim(420, 595)
    plt.xticks(x2)
    plt.yticks(x2)
    plt.tick_params(labelsize=15)
    plt.xlabel('Test value', fontdict=font2)
    plt.ylabel('Predicted value', fontdict=font2)
    plt.show()
    return mses, maes, maes_relative

kernel_1 = 1.0 * RBF(length_scale=1,\
                     length_scale_bounds=(1e-3, 1e3)) +\
WhiteKernel(noise_level=1e-4, noise_level_bounds=(1e-10, 1e+2))
reg_1 = GaussianProcessRegressor(kernel=kernel_1,\
                   n_restarts_optimizer=20,\
                    alpha=0, random_state=42)

mses2, maes2, maes_relative = cv_loop2(reg_1)

-13.412538494550702
-11.97214495008163
-13.810910481789517
-13.767548265459304
-14.60368346492887
-14.92854556779503
-14.814737028759598
-13.370902193873171
-14.715341346510659
-13.464172582444105

png

#output RMSE, MAE, RMD
np.mean(mses2), np.mean(maes2), np.mean(maes_relative)

(17.796925509306437, 15.133874990829066, 0.02990012502880124)

Gaussian Process Regression

noisy kernel function, Gaussian kernel with White noise kernel

kernel_1 = 1.0 * RBF(length_scale=1,\
                   length_scale_bounds=(1e-3, 1e2)) +\
WhiteKernel(noise_level=1e-3, noise_level_bounds=(1e-10, 1e1))
reg_1 = GaussianProcessRegressor(kernel=kernel_1,\
                               n_restarts_optimizer=12,\
                               alpha=0, random_state=42)
reg_1.fit(new_df[['power', 'feed rate']], new_df['microhardness'])

In a Jupyter environment, please rerun this cell to show the HTML representation or trust the notebook. On GitHub, the HTML representation is unable to render, please try loading this page with nbviewer.org.

GaussianProcessRegressor

theta = reg_1.kernel_.k1.theta
noise_level = reg_1.kernel_.k2.noise_level
print('optimized theta: {}\n optimized noise_level: {}'\
     .format(theta, noise_level))

optimized theta: [0.78458968 0.8398592 ]
 optimized noise_level: 0.239298005523145

Export hyperparameters

reg_1.get_params(),reg_1.log_marginal_likelihood()

({'alpha': 0,
  'copy_X_train': True,
  'kernel__k1': 1**2 * RBF(length_scale=1),
  'kernel__k2': WhiteKernel(noise_level=0.001),
  'kernel__k1__k1': 1**2,
  'kernel__k1__k2': RBF(length_scale=1),
  'kernel__k1__k1__constant_value': 1.0,
  'kernel__k1__k1__constant_value_bounds': (1e-05, 100000.0),
  'kernel__k1__k2__length_scale': 1,
  'kernel__k1__k2__length_scale_bounds': (0.001, 100.0),
  'kernel__k2__noise_level': 0.001,
  'kernel__k2__noise_level_bounds': (1e-10, 10.0),
  'kernel': 1**2 * RBF(length_scale=1) + WhiteKernel(noise_level=0.001),
  'n_restarts_optimizer': 12,
  'n_targets': None,
  'normalize_y': False,
  'optimizer': 'fmin_l_bfgs_b',
  'random_state': 42},
 -24.90949851584758)

# create a grid for plotting test dat
p_min, p_max = new_df['power'].min(),\
new_df['power'].max()
s_min, s_max = new_df['feed rate'].min(),\
new_df['feed rate'].max()
print(p_max, p_min)
print(s_max, s_min)
p_set1, s_set1 = np.meshgrid(np.arange(p_min, p_max + 0.3, 0.01),\
                          np.arange(s_min, s_max + 0.3, 0.01))

1.4638501094227996 -1.4638501094227996
1.3416407864998738 -1.3416407864998738

p_set1.shape

(299, 323)

output1, std = reg_1.predict(np.c_[p_set1.ravel(), s_set1.ravel()]\
                         , return_std=True)
output1, std = output1.reshape(p_set1.shape), std.reshape(p_set1.shape)

Upper and lower confidence intervals

up, down = output1 * (1 + 1.96*std), output1 * (1 - 1.96*std)

output1

array([[-0.79011718, -0.77982157, -0.76949433, ...,  1.65955374,
         1.65979042,  1.6599775 ],
       [-0.79393538, -0.78362508, -0.77328318, ...,  1.65560917,
         1.65583558,  1.65601244],
       [-0.79776061, -0.7874358 , -0.7770794 , ...,  1.65161011,
         1.65182623,  1.65199285],
       ...,
       [-1.30023403, -1.29307306, -1.28590472, ..., -0.42134032,
        -0.42346986, -0.42560408],
       [-1.29757646, -1.29044241, -1.2833011 , ..., -0.42577253,
        -0.42790494, -0.4300419 ],
       [-1.29488864, -1.28778153, -1.28066726, ..., -0.43015956,
        -0.4322948 , -0.43443445]])

# lower bound of the Ggaussian confidence interval
ucb_1 = output1 + 0.1*std
ucb_2 = output1 + 1*std
ucb_3 = output1 + 100*std

output1_data = pd.DataFrame(columns=['power', 'feed rate', 'microhardness'])
output1_data['power'] = p_set1.ravel()
output1_data['feed rate'] = s_set1.ravel()
output1_data['microhardness'] = output1.ravel()
output1_data.shape

(96577, 3)

# denormalization of predicted data
scale_X = scaler_X.inverse_transform(output1_data[['power', 'feed rate']])
scale_Y = scaler_Y.inverse_transform(output1_data[['microhardness']])

# variance denormalization
scale_std = scaler_Y.inverse_transform(std)

new_output = pd.DataFrame(columns=['power', 'feed rate', 'microhardness'])
new_output['power'] = scale_X[:, 0].ravel()
new_output['feed rate'] = scale_X[:, 1].ravel()
new_output['microhardness'] = scale_Y.ravel()

p_inverse = scale_X[:, 0].reshape(p_set1.shape)
s_inverse = scale_X[:, 1].reshape(s_set1.shape)
out_inverse = scale_Y.reshape(p_set1.shape)

p_inverse.max()
s_inverse.max()

1.0497611929712034

output_pivot = new_output.pivot(index='power', columns='feed rate', values='microhardness')
f, ax = plt.subplots(figsize=(12,6))
sns.heatmap(output_pivot, ax=ax)
ax.set_title('noise kernel heatmap', fontsize=30)

Text(0.5, 1.0, 'noise kernel heatmap')

png

# plot surface, contour, and combined scatter plot
sns.set(
    style='darkgrid',
    font='Times New Roman',
    font_scale=1.8
)
fig = plt.figure(figsize=(10, 6), facecolor='#ebecf2')
ax = Axes3D(fig)
fig.add_axes(ax)
surface = ax.plot_surface(p_inverse, s_inverse, out_inverse,\
                          cmap=cm.coolwarm, linewidth=0,\
                          alpha=0.8, antialiased=False)
ax.scatter(df['power'], df['feed rate'], df['microhardness'],\
           c=df['microhardness'], cmap=cm.coolwarm)
ax.contourf(p_inverse, s_inverse, out_inverse, np.arange(420, 620, 1),\
            zdir='output1', offset=418, cmap=cm.coolwarm, alpha=0.8)
ax.set_xlabel('\nLaser Power(W)', fontdict=font2)
ax.set_ylabel('\nPowder feed rate(rad/s)', fontdict=font2)
plt.colorbar(surface, shrink=0.9)
plt.tick_params(labelsize=13)
plt.show()

png

Marginal likelihood visualization

plt.figure(figsize=(12, 8))
theta0 = np.logspace(-2, 4, 50)
theta1 = np.logspace(-2, 1, 50)
Theta0, Theta1 = np.meshgrid(theta0, theta1)
LML = [reg_1.log_marginal_likelihood(np.log([0.36, t0, t1]))
        for t0, t1 in zip(Theta0.ravel(), Theta1.ravel())]
LML = np.reshape(LML, newshape=Theta0.shape)
vmin, vmax = (-LML).min(), 50
print(vmax)
print(vmin)
level = np.around(np.logspace(np.log10(vmin), np.log10(vmax), num=50), decimals=1)
plt.contour(Theta0, Theta1, -LML, level, cmap=cm.coolwarm)
plt.colorbar()
plt.xscale("log")
plt.yscale("log")
plt.xlabel("Length-scale")
plt.ylabel("Noise-level")
plt.title("Log-marginal-likelihood")
plt.tight_layout()
plt.show()

50
27.114380349856493

png

# surface visualization
x = p_inverse.ravel()
y = s_inverse.ravel()
z = out_inverse.ravel()
point_obj = zip(x, y, z)
data = []
for point in point_obj:
    data.append(point)
len(data)

96577

sns.set_style('white')
font1 = {
    'family': 'Times New Roman',
    'weight': 'bold',
    'size': 30
}
font2 = {
    'family': 'Times New Roman',
    'weight': 'normal',
    'size': 15
}
font3 = {
    'family': 'Times New Roman',
    'weight': 'normal',
    'size': 30
}

# plot contour map
fig, ax = plt.subplots(figsize=(12,6))
plt.contourf(p_inverse, s_inverse,\
             out_inverse, np.arange(420, 620, 1),\
                   offset=0, cmap=cm.coolwarm,\
             alpha=0.85)
sns.scatterplot(x='power', y='feed rate', size='microhardness',\
               data=df, sizes=(10, 100))
plt.legend(loc='upper left', prop=font2)
plt.tick_params(labelsize=24)
labels = ax.get_xticklabels() + ax.get_yticklabels()
[label.set_fontname('Times New Roman') for label in labels]
cb = plt.colorbar()
cb_labels = cb.ax.yaxis.get_ticklabels()
for cb_label in cb_labels:
    cb_label.set_family('Times New Roman')
    cb_label.set_fontsize(18)
x_tick = [700 + i*100 for i in range(6)]
y_tick = [0.55 + j*0.15 for j in range(6)]
plt.xticks(x_tick, fontsize=16)
plt.yticks(y_tick, fontsize=16)
plt.xlim(700, 1250)
plt.ylim(0.55, 1.05)
plt.title('Microhardness Contour Distribution', fontdict=font2)
plt.xlabel('Laser Power (W)', fontdict=font2)
plt.ylabel('Feed rate (rad/min)', fontdict=font2)
plt.show()

png

out_inverse.min(),out_inverse.max()

(469.9414283576257, 568.9217848129492)

# plot contour map
plt.figure(figsize=(12,6))
contour = plt.contour(p_inverse, s_inverse, out_inverse,\
            np.arange(out_inverse.min(), out_inverse.max(), 1),
                      off_set=0, cmap=cm.coolwarm, alpha=0.7)
plt.colorbar()
plt.title('Microhardness Contour Distribution', fontdict=font2)
plt.xlabel('power', fontdict=font2)
plt.ylabel('feed rate', fontdict=font2)
plt.clabel(contour)
plt.tick_params(labelsize=24)
plt.show()

png

UCB point selection

# upper bound of the confidence interval
#β values of 0.1, 1, 10, 100
ucb_1 = out_inverse + 0.01*std
ucb_2 = out_inverse + 0.1*std
ucb_3 = out_inverse + 1*std
ucb_4 = out_inverse + 10*std

# calculate UCB data
output1_data = pd.DataFrame(columns=['power', 'feed rate', 'microhardness'])
output1_data['power'] = p_inverse.ravel()
output1_data['feed rate'] = s_inverse.ravel()
output1_data['microhardness'] = out_inverse.ravel()
output1_data['ucb-0.01'] = ucb_1.ravel()
output1_data['ucb-0.1'] = ucb_2.ravel()
output1_data['ucb-1'] = ucb_3.ravel()
output1_data['ucb-10'] = ucb_4.ravel()
output1_data.head()

power

feed rate

microhardness

ucb-0.01

ucb-0.1

ucb-1

ucb-10

0

700.000000

0.55

489.764768

489.770561

489.822696

490.344048

495.557565

1

701.707825

0.55

490.097396

490.103178

490.155222

490.675655

495.879985

2

703.415650

0.55

490.431045

490.436818

490.488771

491.008301

496.203606

3

705.123475

0.55

490.765701

490.771464

490.823328

491.341972

496.528408

4

706.831301

0.55

491.101347

491.107100

491.158878

491.676650

496.854377

a = (p_inverse.min() + p_inverse.max())/2
b = (s_inverse.min() + s_inverse.max())/2

x1, y1, x2, y2 = [], [], [], []
for i in range(11):
    x1.append(a)
    y1.append(600 + 112*i)
    x2.append(100 + 26*i)
    y2.append(b)

a,b

(974.9598455532495, 0.7998805964856017)

# divide regions based on UCB for point selection
temp1 = output1_data[(output1_data['power'] <= a) &\
                    (output1_data['feed rate'] <= b)]
temp2 = output1_data[(output1_data['power'] > a) &\
                    (output1_data['feed rate'] <= b)]
temp3 = output1_data[(output1_data['power'] <= a) &\
                    (output1_data['feed rate'] > b)]
temp4 = output1_data[(output1_data['power'] > a) &\
                    (output1_data['feed rate'] > b)]

ucb_res = pd.DataFrame(columns=['power', 'feed rate', 'microhardness', 'ucb'])
temp = [output1_data]
ucb = ['ucb-0.01', 'ucb-0.1', 'ucb-1', 'ucb-10']
k = 1
for t in temp:
    for u in ucb:
        sort_data = t.sort_values(by=u, ascending=False)
        filter_data = sort_data[['power', 'feed rate', 'microhardness', u]].iloc[0:k]
        index = [u]
        filter_data.index = index
        filter_data.columns = ucb_res.columns
        ucb_res = pd.concat([ucb_res, filter_data])
        print(ucb_res)

power  feed rate  microhardness         ucb
ucb-0.01  1249.919691       0.55     568.921785  568.927959
                power  feed rate  microhardness         ucb
ucb-0.01  1249.919691       0.55     568.921785  568.927959
ucb-0.1   1249.919691       0.55     568.921785  568.983526
                power  feed rate  microhardness         ucb
ucb-0.01  1249.919691       0.55     568.921785  568.927959
ucb-0.1   1249.919691       0.55     568.921785  568.983526
ucb-1     1249.919691       0.55     568.921785  569.539194
                power  feed rate  microhardness         ucb
ucb-0.01  1249.919691       0.55     568.921785  568.927959
ucb-0.1   1249.919691       0.55     568.921785  568.983526
ucb-1     1249.919691       0.55     568.921785  569.539194
ucb-10    1249.919691       0.55     568.921785  575.095876

ucb_data = ucb_res
ucb_data['ucbβ'] = ucb_data.index
ucb_data = ucb_data[['ucbβ', 'power', 'feed rate', 'microhardness', 'ucb']]
ucb_data.shape

(4, 5)

# write UCB selected data
save_path =r'C:\Users\Administrator\Desktop\3.xls'
size = ucb_data.shape
work_book = xlwt.Workbook(encoding='utf-8')
work_sheet = work_book.add_sheet('_ucb')
columns = list(ucb_data.columns)
for i in range(size[1]):
    work_sheet.write(0, i, columns[i])
for i in range(size[0]):
    for j in range(size[1]):
        work_sheet.write(i+1, j, ucb_data.iloc[i, j])
work_book.save(save_path)
print('Done!')

Done!

# visualize and compare UCB selected points
# plot contour map 
sns.set_style('white')
fig, ax = plt.subplots(figsize=(10,6))
plt.contourf(p_inverse, s_inverse,\
             out_inverse,  np.arange(420, 620, 2),\
                   offset=0, cmap=cm.coolwarm,\
             alpha=0.8)
p1 = sns.scatterplot(x='power', y='feed rate', size='microhardness',\
                hue='ucbβ',\
                     hue_order=['ucb-0.01', 'ucb-0.1', 'ucb-1', 'ucb-10'],\
                     sizes=(20, 100), data=ucb_data, legend=False)
plt.tick_params(labelsize=13)
labels = ax.get_xticklabels() + ax.get_yticklabels()
[label.set_fontname('Times New Roman') for label in labels]
cb = plt.colorbar()
cb_labels = cb.ax.yaxis.get_ticklabels()
for cb_label in cb_labels:
    cb_label.set_family('Times New Roman')
    cb_label.set_fontsize(13)
x_tick = [600 + i*100 for i in range(7)]
y_tick = [0.55 + j*0.15 for j in range(4)]
plt.xticks(x_tick, fontsize=13)
plt.yticks(y_tick, fontsize=13)
plt.xlim(700, 1250)
plt.ylim(0.55, 1.05)
plt.plot(x1, y1)
plt.plot(x2, y2, 'r')
plt.title('Microhardness UCB', fontdict=font2)
plt.xlabel('Laser power(W)', fontdict=font2)
plt.ylabel('Powder feed rate(rad/s)', fontdict=font2)
plt.show()

png

Variance visualization

sns.set_style('darkgrid')
fig1 = plt.figure(figsize=(12,10))
ax1 = fig1.add_subplot(111, projection='3d')
surf = ax1.plot_surface(p_inverse, s_inverse, std, rstride=8,\
                        cstride=3, linewidth=0, alpha=0.4,\
                        antialiased=True, cmap=cm.plasma)
ax1.set_title('Microhardness Variance', fontdict=font2)
ax1.set_xlabel('\nLaser power(W)', fontdict=font2)
ax1.set_ylabel('\nPowder feed rate(rad/s)', fontdict=font2)
plt.colorbar(surf, shrink=0.9)
plt.tick_params(labelsize=13)
plt.show()

png

std.min()

0.5217224756540765